The patient was brought to the catheterization laboratory, intubated by the anesthesiology team, and maintained in 21% FiO2. They were prepped and draped in the standard sterile fashion. The patient was re-identified and procedural time-out completed.

Vascular access was obtained using the modified Seldinger technique with ultrasound guidance. A __ sheath was placed in the ___ femoral vein and the 2 Fr inner dilator of a 401 set placed in the ___ femoral artery. An opening saturation, blood gas, and ACT were obtained. ___ units (~ 100 units/kg) of unfractionated heparin were administered.

A __ thermodilution catheter was advanced to the SVC, and then to the RA, RV, and __PA to measure saturations and pressures, including a __ pulmonary capillary wedge pressure. Cardiac index was measured by thermodilution.

The patient was then placed on 100% FiO2 and 40 ppm of inhaled nitric oxide. After a 5-minute wash-in period, a __PA saturation and an arterial saturation and blood gas were remeasured. Cardiac index was remeasured by thermodilution. A pressure pullback was performed from the __ pulmonary capillary wedge pressure to the RA.

The patient tolerated the procedure well without complications. Lines were removed and hemostasis achieved with the application of light pressure. The patient was transported to the ___ for post-procedure monitoring and recovery. The findings were discussed with the family and treating physicians.